Towards long-wave infrared narrowband tunable FPIs

Gurpreet Singh Gill1*, Michal Zawierta1, Dhirendra Kumar Tripathi1, Adrian Keating2, Gino Putrino1, Dilusha Silva1, Lorenzo Faraone1,3 and Mariusz Martyniuk1,3

1 Department of Electrical, Electronic and Computer Engineering, 2 Department of Mechanical Engineering, 3ARC Centre of Excellence on Transformative Meta-Optical Systems (TMOS); The University of Western Australia, Perth, WA – 6009, Australia

*gurpreet.gill@uwa.edu.au

Abstract: This work presents a surface micromachined long-wave infrared (LWIR) tunable Fabry–Pérot interferometer (FPI) incorporating Ge/BaF2/Ge solid-material distributed Bragg’s reflectors (DBRs) for 8 – 10 µm optical wavelength range. Ge and BaF2 thin film based static FPIs were integrated with a tuning mechanism to realise wavelength discrimination. This has been achieved using two different approaches to fabricate the tuneable cavity FPIs (δ- and ε-series), where the bottom high-index Ge layer of the top DBR was targeted to be either partially (in the case of δ-series FPIs) or fully (in the case of ε-series FPIs) replaced with a conformal Si layer in order to provide structural support to the top DBR. The fabricated FPIs exhibited peak transmission ranging from 20 – 25%, full width at half maxima (FWHM) ranging from 190 – 436 nm for the centre wavelength tuning from ~9.93 µm to ~7.98 µm. This work represents a reliable and reproducible fabrication process for tunable cavity LWIR FPIs.

Keywords: Fabry–Pérot interferometer, distributed Bragg’s reflectors, thin films, surface micromaching, long-wave infrared.

Transmission tuning spectra of tuneable cavity FPIs (δ- and ε-series). The transmission response is plotted linearly as percentage for FPIs that are 500um in latheral dimension for (a) δ and (b) ε-series. The data points represent the measured FPIs transmission spectra at different DC voltages, whereas, the solid lines represent the modelled FPIs performance for an unactuated FP cavity.